CPE301 – SPRING 2019

# Design Assignment 6

Student Name: Chris Barr

Student #: 2000682859

Student Email: barrc1@unlv.nevada.edu

Primary Github address: https://github.com/BarrChris

Directory: https://github.com/BarrChris/submission_da.git

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

- Atmega328P

- 4 M-F wires

- MPU 6050

Block diagram with pins used in the Atmega328P

SCL was connected to the PC5

SDA was connected to the PC4

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <util/delay.h>

#include <math.h>

#include <stdlib.h>										/* Include standard library file */

#include <stdio.h>										/* Include standard library file */

#include "MPU6050_def.h"							/* Include MPU6050 register define file */

#include "i2c_master.h"							/* Include I2C Master header file */

#include "uart.h"							/* Include USART header file */

#define MPU6050_WRITE 0xD0

#define MPU6050_READ 0xD1

float Acc_x;

float Acc_z;

float Acc_y;

float Gyro_x;

float Gyro_z;

float Gyro_y;

void init_uart(uint16_t baudrate){

uint16_t UBRR_val = (F_CPU/16)/(baudrate-1);

UBRR0H = UBRR_val >> 8;

UBRR0L = UBRR_val;

UCSR0B |= (1<<TXEN0) | (1<<RXEN0) | (1<<RXCIE0); // UART TX (Transmit - senden) einschalten

UCSR0C |= (1<<USBS0) | (3<<UCSZ00); //Modus Asynchron 8N1 (8 Datenbits, No Parity, 1 Stopbit)

}

void uart_putc(unsigned char c){

while(!(UCSR0A & (1<<UDRE0))); // wait until sending is possible

UDR0 = c; // output character saved in c

}

void uart_puts(char *s){

while(*s){

uart_putc(*s);

s++;

}

}

void init_MPU6050(void){

_delay_ms(150);										/* Power up time >100ms */

i2c_start(MPU6050_WRITE); // Set Gyroscope Sample Rate = 1 KHz, Accelerometer Sample Rate = 1 KHz (default)

i2c_write(SMPLRT_DIV); // Sample Rate is generated by dividing the gyroscope output rate by SMPLRT_DIV

i2c_write(0x07); // Gyroscope Output Rate = 8kHz, Sample Rate = Gyroscope Output Rate / (1 + SMPLRT_DIV)

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(PWR_MGMT_1);

i2c_write(0x01); // PLL with X axis gyroscope reference

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(CONFIG); //Frame Synchronization & Digital Low Pass Filter (DLPF) setting

i2c_write(0x00);

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(GYRO_CONFIG); //gyroscopes’ scale range = FS_SEL selects = 11 = ± 2000 °/s

i2c_write(0x18);		// accelerometer range = ± 2g (default)

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(INT_ENABLE); // DATA_RDY_EN = 1

i2c_write(0x01);

i2c_stop();

}

void getreading(void){

i2c_start(MPU6050_WRITE);

i2c_write(ACCEL_XOUT_H); // set pointer

i2c_write(ACCEL_YOUT_H); // set pointer

i2c_write(ACCEL_ZOUT_H); // set pointer

i2c_stop();

i2c_start(MPU6050_READ);

Acc_x = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

Acc_y = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

Acc_z = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

i2c_stop();

//=========================================================

i2c_start(MPU6050_WRITE);

i2c_write(GYRO_XOUT_H);

i2c_write(GYRO_YOUT_H);

i2c_write(GYRO_ZOUT_H);

i2c_stop();

i2c_start(MPU6050_READ);

Gyro_x = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

Gyro_y = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

Gyro_z = (((int)i2c_read_ack()<<8) | (int)i2c_read_nack());

i2c_stop();

}

int main(void){

char buffer[20], float_[10];

float Xa, Ya, Za;

float Xg, Yg, Zg;

init_uart(9600);

i2c_init();

init_MPU6050();

while(1){

getreading();

Xa = Acc_x/16384.0;								/* Divide raw value by sensitivity scale factor to get real values */

Ya = Acc_y/16384.0;

Za = Acc_x/16384.0;

Xg = Gyro_x/16.4;								/* Divide raw value by sensitivity scale factor to get real values */

Yg = Gyro_y/16.4;

Zg = Gyro_z/16.4;

dtostrf( Xa, 3, 2, float_ );					/* Take values in buffer to send all parameters over USART */

sprintf(buffer,"Ax: %s, ",float_);

USART_SendString(buffer);

dtostrf( Ya, 3, 2, float_ );

sprintf(buffer,"Ay: %s, ",float_);

USART_SendString(buffer);

dtostrf( Za, 3, 2, float_ );

sprintf(buffer,"Az: %s, \n",float_);

USART_SendString(buffer);

dtostrf( Xg, 3, 2, float_ );

sprintf(buffer,"Gx: %s, ",float_);

USART_SendString(buffer);

dtostrf( Yg, 3, 2, float_ );

sprintf(buffer,"Gy: %s, ",float_);

USART_SendString(buffer);

dtostrf( Zg, 3, 2, float_ );

sprintf(buffer,"Gz: %s, \n\n",float_);

USART_SendString(buffer);

_delay_ms(1000);

}

return 0;

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=q2rA5xOTPCk

- GITHUB LINK OF THIS DA

https://github.com/BarrChris/submission_da/tree/master/DesignAssignments/DA6

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Chris Barr